Preliminary code for BeStable platform

Figure 1: BeStable platform

- State machine

Figure 2: State machine diagram

Descriptions

- Start: The default state when D-Flow is started with the “Start” action button from the Runtime Console. This automatically transitions to the Initialize state.

- Initialize: Set default internal parameters. Once successful, this automatically transitions to the Free state.

- Free: Walk without targets shown

- Perturb: A target is shown on the walking surface of the treadmill.

- Record the following parameters every step:

  - step number

  - time stamp

  - initial limb

  - final limb

  - step length,

  - step width,

  - step time (duration),

  - target error (after disappearing visual cue)

  - message of the step characteristic

- Stop: The default state when D-Flow is stopped with the “Stop” action button.

- User interface

Figure 3: User interface – input subject characteristics

Figure 4: User interface – select and set perturbation parameters

- Code

All codes running on BeStable platform are attached as the following separate files:

| File name | Description |
| --- | --- |
| bestable_main.lua | main function, state machine |
| general.lua | basic mathematic functions |
| gaitDetectionGTXLib.dll | library for IMU |
| gtxLib.lua | functions for IMU |
| objectPlacement.lua | functions for visual cue placement and sounds |
| protocol.lua | functions for creating perturbations |
| bestable.lua | additional functions, moving average, writing output data |
| uriLib.lua | gait detection algorithm from CoP |

- An example of the output file

The example of the output_file.csv is attached, where subject characteristics (first two lines) and data from the measurements are stored. The following shows an example of the output file:

Table 1: An example of the output_file.csv from the BeStable platform.

| ID | age | height | mass | gender | affectedSide |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 37 | 1.8 | 75 | 1 | 3 |  |  |  |
| stepNumber | timeStamp | limbInitial | limbFinal | stepWidth | stepLength | stepTime | targetError | message |
| 1 | 14.62338532 | R | R | 0.019934174 | 0.045019359 | 0.04 | -1 | free |
| 2 | 15.23005198 | R | L | 0.00722311 | 0.063710654 | 0.093333333 | -1 | free |
| 3 | 15.93338532 | L | R | 0.252779696 | 0.472348443 | 0.796666667 | -1 | free |
| … | … | … | … | … | … | … | … | … |
| 32 | 49.53535111 | L | R | 0.191800449 | 0.511645719 | 0.783333333 | -1 | free |
| 33 | 50.43868444 | R | L | 0.19222166 | 0.478648364 | 0.716666667 | -1 | Pert: fwiw (R) |
| 34 | 51.35535111 | L | R | 0.341273615 | 0.817996197 | 1.023333333 | -1 | action |
| 35 | 52.08535111 | R | L | 0.347576107 | 0.272384992 | 0.69 | 0.015461387 | target hit |
| 36 | 52.85245746 | L | R | 0.180584256 | 0.480551683 | 0.85 | -1 | free |
| 37 | 53.63846364 | R | L | 0.167030066 | 0.501109838 | 0.772108089 | -1 | free |
| 38 | 54.48972629 | L | R | 0.166231278 | 0.481760327 | 0.834337778 | -1 | free |
| 39 | 55.22972629 | R | L | 0.169267375 | 0.473764102 | 0.771262649 | -1 | free |
| 40 | 56.05305963 | L | R | 0.164395532 | 0.487504169 | 0.81 | -1 | free |
| 41 | 56.80957504 | R | L | 0.159972916 | 0.468546106 | 0.753333333 | -1 | free |
| 42 | 57.60624171 | L | R | 0.163537153 | 0.528303927 | 0.839848747 | -1 | free |
| 43 | 58.36290837 | R | L | 0.152343318 | 0.469841308 | 0.753333333 | -1 | free |
| 44 | 59.17274439 | L | R | 0.16651024 | 0.470625822 | 0.796502684 | -1 | free |
| 45 | 59.90098645 | R | L | 0.169050727 | 0.47556727 | 0.73 | -1 | free |
| 46 | 60.83440014 | L | R | 0.172413276 | 0.451774603 | 0.768242062 | -1 | Pert: fw (L) |
| 47 | 62.07921998 | R | L | 0.152264081 | 0.906792767 | 1.055711787 | -1 | action |
| 48 | 62.69588665 | L | R | 0.182199787 | 0.277610428 | 0.875855076 | 0.00857886 | target hit |
| 49 | 63.46588665 | R | L | 0.200627622 | 0.535225351 | 0.863333333 | -1 | free |
| 50 | 64.23588665 | L | R | 0.197666949 | 0.436199215 | 0.796666667 | -1 | free |
| 51 | 65.00588665 | R | L | 0.192569566 | 0.497996065 | 0.743333333 | -1 | free |
| 52 | 65.78921998 | L | R | 0.175014517 | 0.452498054 | 0.77 | -1 | free |
| 53 | 66.52921998 | R | L | 0.177355289 | 0.489733235 | 0.77 | -1 | free |
| 54 | 67.29921998 | L | R | 0.166294421 | 0.461086 | 0.756666667 | -1 | free |
| 55 | 68.37921998 | R | L | 0.175160002 | 0.473433837 | 0.78 | -1 | Pert: fwow (R) |
| 56 | 69.36588665 | L | R | 0.190616928 | 0.837045781 | 1.15 | -1 | action |
| 57 | 70.17588665 | R | L | 0.200079409 | 0.476791853 | 0.903333333 | 0.178416793 | target missed |
| 58 | 70.90588665 | L | R | 0.189284172 | 0.441820289 | 0.77 | -1 | free |
| 59 | 71.78255332 | R | L | 0.156163852 | 0.482645907 | 0.81 | -1 | free |
| 60 | 72.56588665 | L | R | 0.172234126 | 0.455091758 | 0.876666667 | -1 | free |
| etc... |  |  |  |  |  |  |  |  |

Figure 5: Initial and final limb

Target location is visualized as a blue rectangle (for the right leg) or red rectangle (for the left leg). Target error is calculated as a minimum distance between CoP and target position, i.e. center of rectangle, in a time window when target is visible. The target is displayed at the heel strike of the opposite leg and it disappears one gait cycle after.

- Script for signal post-processing

The data from the measurement is segmented to the following structures:

- BASE walking: unperturbed walking before enabling perturbations;

- FREE walking: unperturbed walking between consecutive perturbations;

- PERTURBATIONS: all perturbations are sorted based on their directions:

    - fw as forward direction,

    - iw as inward direction,

    - fwiw as forward-inward direction,

    - ow as outward direction,

    - fwow as forward-outward direction,

and target foot:

- (L) as visual target for left foot (red rectangle), and

- (R) as visual target for right foot (blue rectangle).

Step width, length and time of four consecutive steps after perturbation onset are stored to the perturbation structure.

- Visualization of the measurement

The report of each measurement can be displayed as shown in Figure 6.

Figure 6: Presentation of the measured data of the subject compared to the group normative.